What we do today is to ensure the future of our

group with youth and spirit forever and ever.

- SBM

Shanghai Shibang Machinery Co., Ltd

2

HCS 90 Cone Crusher

HCS 90 Cone Crusher Overview

The HCS90 type cone crusher with high-efficiency and single cylinder hydraulic

pressure applies to metal mine , metallurgical, chemical , building, cement and

sandstone industry etc. It can Apply to the middle and smallbroken bitsbreak

materious of mines and iron ore with the Knoop number hardness f = mine

5-16's,such as sums rock, non-ferrous metal ore , granite , limestone , quartzite ,

sandrock , cobblestone etc.

www.unisbm.com

Shanghai Shibang Machinery Co., Ltd

3

Applications

Spring cone crushers are widely used in industries like metallurgy, construction,

road building, chemistry etc, to crush many kinds of mid-hard or hard rocks and

ores such as iron ore, limestone, granite, basalt, quartz etc. The standard ones are

used for intermediate crushing; the medium ones are used for fine crushing; and

the short head ones are used for super thin crushing.

Main Features and Benefits

The HCS90 type cone crusher with high-efficiency and single cylinder hydraulic

pressure is a kind of high-tech crusher to meet the requirements of developed

countries.it is designed by our company who brings in the USA technology , and

combines with the China metal material function.The characters of this mechine are

as following:

•High efficiency: the main axle have support both on the high and low sides which

can stand more stave strength and stroke.In addition to special stave cavity shape

which adapts with the lamination principle, enables this machine has higher

crushing efficiency.

•High production: This machine is designed consummatly which combined the will

break stroke , the broken speed and the feture of broken cavity ,so that under the

same moving identical cone big end diameter condition ,the output is higher than

the old-fashioned damaged spring cone output about 35% ~60%.

•High-efficiency cobblestone: This machine adopts the proper broken cavity shape

and broken principle of lamination , produces broken effect among particles,thereby

increases the cube proportion of finished product obviously, decreases the needle

schistose cobblestone, and the grade is more homogeneous.

•It only needs to change the fix cone to meet the needs of various cavity middle and

thin break .

•Double assurances control the hydraulic pressure and lubricating system to ensure

the overload protection and fine axle bearing lubricates.

•Covenient maintenance: The structure this machine is simple but compact , which

have a covenient maintenance.

www.unisbm.com

Shanghai Shibang Machinery Co., Ltd

4

Technical Data

1. technology parameter

Feeding widden（mm）

Max feeding widden（mm）

Discharging range（mm）

Power    （Kw）

Max permitting dimension（mm）

Overall dimension length×widden×hight（mm）

weight（t）

2. lubricate pressure system parameter

Oil motor power（Kw）

Oil motor pressure（MPa）

Oil box cubage（L）

3. hydranlic pressure system parameter

Oil motor power（Kw）

Oil motor pressure（MPa）

Oil box cubag（L）

4. capacity

① Model A crushing cavity

discharge（mm）

production（t/h）

16

65-75

18

70-85

135（100、80、50）four types

110（80、65、40）

6-28

（75~110）

<40

2581×1550×3276

9.5

1.5

16

100

2.2

1.0

800

22

85-105

28

110-130

② Model B crushing cavity

discharge（mm）

production（t/h）

11

45-55

13

55-65

16

65-75

20

80-100

③ Model C crushing cavity

discharge（mm）

production（t/h）

9

25-35

11

40-50

13

50-60

16

60-70

④ Model D crushing cavity

discharge（mm）

production（t/h）

6

20-25

7

25-30

8

30-35

10

35-45

Note: Capacity is measured by the local quartz stone. The broken ability is affected

by various factor, such as raw material hardness , humidity , size collocation ,

operating method , discharger size etc. This form only for reference.

www.unisbm.com

Shanghai Shibang Machinery Co., Ltd

5

How HCS 90 Cone Crusher Works

The motor drives the eccentric shaft bushing through V-belt, transmission shaft,

and transmission gear. The cone makes circle movement at the force of eccentric

shaft bushing, and close to mantle from time to time. Raw materials are crushed

during the process and are later discharged at the bottom.

www.unisbm.com

Shanghai Shibang Machinery Co., Ltd

6

Sales Hot-Line

International Dept.1: Tel: 0086-21-58383098

International Dept.2: Tel: 0086-21-58383028

International Dept.3: Tel: 0086-21-58383058

International Dept.4: Tel: 0086-21-58383003

International Dept.5: Tel: 0086-21-58383022

E-mail & Fax

E-mail:

Fax:

sbm@unisbm.com

0086-21-58383058

Office Address & Zip

Add : No.416 Jianye Road,South Jinqiao Area,Pudong, Shanghai, China

Zip / Postal Code : 201201

www.unisbm.com