What we do today is to ensure the future of our

group with youth and spirit forever and ever.

- SBM

Shanghai Shibang Machinery Co., Ltd

2

Spring Cone Crusher

Spring Cone Crusher Overview

With advantages of reliable construction, high prod activity, easy adjustment and

low cost in operation, SBM spring cone crusher is suitable for crushing various of

mid-hard and above mid-hard ores and rocks. The spring release system of SBM

cone crusher acts an overload protection system that allows chunk to pass through

the crushing chamber without damage to the crusher, two kinds of sealed formation

dry oil and water are used. It separates plaster stone and engine oil, reliable

performance ensured.

www.unisbm.com

Shanghai Shibang Machinery Co., Ltd

3

Applications

Spring cone crushers are widely used in industries like metallurgy, construction,

road building, chemistry etc, to crush many kinds of mid-hard or hard rocks and

ores such as iron ore, limestone, granite, basalt, quartz etc. The standard ones are

used for intermediate crushing; the medium ones are used for fine crushing; and

the short head ones are used for super thin crushing.

Main Features and Benefits

1. High productivity; high quality

2. Less machine stop time

3. Easy maintenance and low cost

4. Unique crushing ability in primary, secondary and tertiary crushing

Technical Data

Model

PYB

PYZ

PYD

PYB

PYZ

PYD

PYB

PYZ

PYD

PYB

PYZ

PYD

900

1200

1750

2200

Max.

Feed

Size(mm)

115

60

50

145

100

50

215

185

85

300

230

100

Adjustable

Range of

Discharging

Opening(mm)

15-50

5-20

3-13

20-50

8-25

3-15

25-50

10-30

5-13

30-60

10-30

5-15

Motor

Diameter of

Capacity

(t/h)

Power        Cone Crusher at

(kW)              Big End(mm)

Weight

(t)

60-105

50-90

15-50

110-168

42-135

18-105

280-480

115-320

75-230

590-1000

200-580

125-350

55

Ф900

11.2

130

Ф1200

35

160

Ф1750

50.3

50.3

50.2

260

280

Ф2200

80

Note: This specification is just reference, any changes are subject to the    Spring

Cone Crusher products.

www.unisbm.com

Shanghai Shibang Machinery Co., Ltd

4

How Spring Cone Crusher Works

During operation, the motor of Spring Cone Crusher Crushes drives the eccentric

shaft shell to turn around through ahorizontal axle and a pair of bevel gear. The axle

of crushing cone swings with the force of eccentric shaft shell so that the surface of

the crushing wall is close to the roll mortar wall from time to time. In this way, the

ores and rocks will be pressed and crooked and crushed.

www.unisbm.com

Shanghai Shibang Machinery Co., Ltd

5

Sales Hot-Line

International Dept.1: Tel: 0086-21-58383098

International Dept.2: Tel: 0086-21-58383028

International Dept.3: Tel: 0086-21-58383058

International Dept.4: Tel: 0086-21-58383003

International Dept.5: Tel: 0086-21-58383022

E-mail & Fax

E-mail:

Fax:

sbm@unisbm.com

0086-21-58383058

Office Address & Zip

Add : No.416 Jianye Road,South Jinqiao Area,Pudong, Shanghai, China

Zip / Postal Code : 201201

www.unisbm.com